# MP3 Report

Leo Liao

Source Code:

Video:

## Overview

For this miniproject I built the fifth variant from the assignment: three independent Games of Life running on the same 8×8 grid, one for each color of the WS2812B LED matrix. Each LED keeps an 8-bit brightness for red, green and blue. When a cell is alive in a game, the brightness of that color goes up by 32 (stopping at 255); when it is dead, it goes down by 3 (stopping at 0). So a cell lights up quickly when it is born and fades out slowly after it dies, and cells that are alive in more than one game show mixed colors.

The grid uses cyclic boundary conditions, so every cell in the LED matrix has exactly eight neighbors. The games move forward one generation per second, and the matrix is refreshed 32 times per second to adjust the brightness of each cell. The three games start from three classic patterns, read from .mem files with $readmemh: a glider (red), a lightweight spaceship (green) and a toad (blue).

The design targets the iCE40UP5K FPGA on the iceBlinkPico board, clocked by its 12 MHz oscillator. It is written in SystemVerilog, and the Game of Life logic is tested with Icarus Verilog using game_tb.sv.

## System architecture

Figure 1. System block diagram.

The state of each game is a 64-bit register in top.sv, where bit 8×row + column is one cell (because it's a flat vector instead of a matrix). Three game modules compute the next generation of each game. The brightness values are stored in three small memories, one per color, with one 8-bit entry per LED.

The controller from the course example walks through the 64 LEDs one by one, 32 times per second. For each LED it first reads the old brightness from the three memories, then led_cell turns the old brightness and the alive bits of that LED into the new brightness. The new value is written back to memory.sv and loaded into a 24-bit shift register, and ws2812b.sv sends it to the matrix one bit at a time. So every LED gets its brightness updated exactly once per frame.

## Module descriptions

### game.sv

The game module is pure combinational logic. Its input is the 64-bit state of one game and its output is the 64-bit next_state. I used a nested generate for loop over rows and columns, which makes 64 copies of the same small circuit. Each copy finds the rows above and below and the columns to the left and right with (r + 7) % 8 and (r + 1) % 8, which handles the wrap-around, adds up its eight neighbors into a 4-bit count n, and applies the rules:

next = (n == 2 && alive) || n == 3

This one line covers all four rules: a live cell survives with two or three neighbors, a dead cell is born with exactly three, and every other case gives a dead cell.

### cell.sv

The cell module holds the brightness rule. It takes the old red, green and blue brightness (r_old, g_old, b_old) and the three alive bits, and gives the new brightness (level_r, level_g, level_b). It is also combinational. Before adding or subtracting, it checks whether the result would go past 255 or below 0, and if so it stays at 255 or 0 instead of wrapping around.

The step sizes come from three parameters, so I can change how fast things fade without touching the logic: one generation is 32 updates, a dead cell should fade out within 3 generations, and a new cell should reach full brightness in 8 updates. This gives a step of 32 up and 3 down.

### memory.sv

memory stores 64 brightness values of 8 bits, one per LED, and starts with all of them at 0. It has one address, a write enable and a write data input. On each clock edge it writes when write_enable is high and reads otherwise, so the read data is ready one clock after the address is given. top uses three of them, one for each color.

### controller.sv and ws2812b.sv

These two files come unchanged from the course example in iceBlinkPico/examples/led_matrix. controller gives the current LED number pixel, the frame number frame (0 to 31), and the control signals for each LED: one clock to read, one clock to load the shift register (load_sreg), and 24 × 15 clocks to send. After the 64th LED it keeps the data line low until the next frame, which also gives the LEDs the reset time they need. ws2812b turns each bit into a pulse that is high for 5 clocks for a 0 or 10 clocks for a 1, out of 15 clocks per bit.

### top.sv

top connects everything and is the module that goes onto the FPGA. It has one controller, one ws2812b, three game, three memory and one led_cell. The LED number pixel is the address of all three memories, and it also picks the alive bits state_r[pixel], state_g[pixel] and state_b[pixel] for led_cell.

On the first clock after power-up, top copies the three seeds into the state registers. After that, when the last LED of frame 31 is loaded, all three games move to their next generation, which happens once per second. The new brightness is loaded into the shift register as {green, red, blue}, the order the WS2812B expects. The output _48b drives the data line of the matrix.

My first version gave every cell its own brightness registers and updated all 64 cells at once. That needed 9441 logic cells, but the iCE40UP5K only has 5280, so nextpnr could not place it. Moving the brightness into memory and updating one LED at a time, while the controller is already reading it, brought the design down to 2441 logic cells (46%) plus 3 of the 30 block RAMs.

### seed_r.mem, seed_g.mem, seed_b.mem

Each file has 8 lines, one per row, written in hex. Bit 0 of each line is column 0. I picked one pattern of each kind so the three colors move differently: the glider moves diagonally and comes back to its start after 32 generations on the 8×8 wrap-around grid, the lightweight spaceship moves sideways and comes back after 16, and the toad stays in place and flips between two sdhapes.

### game_tb.sv

game_tb tests game on its own, since that is where the rules are. It does three things:

- Blinker. It puts three cells in a row and checks that the next state is the same three cells standing up, compared against the value I worked out by hand.

- Glider. It runs a glider for 32 generations by feeding next_state back into state, and checks that the grid is back to where it started. This only works if the rules and the wrap-around are both right.

- Random states. It gives game 1000 random 64-bit states. For each one the test bench works out the expected next state by itself, written straight from the four rules in the assignment with its own neighbor loop, and counts how many times game disagrees.

## Simulation results

The test bench output is below. Both pattern tests pass, and the 1000 random states give 0 errors.

state      = 0000000000000000

next_state = 0000000000000000

state = 000000000e000000

next_state = 0000000404040000

Pass

initial_state = 0000000000070402

state = 0000000000070402

Pass

Checked 1000 times, error: 0

The first two lines show that an empty grid stays empty. The blinker input 0e000000 is row 3, columns 1 to 3, and the output 0404040000 is column 2 in rows 2, 3 and 4, which is the blinker turned upright. After 32 generations the glider is back at 0000000000070402, its starting position.

To make sure the random test can actually catch mistakes, I also broke game.sv on purpose. Changing n == 3 to n == 4 made all 1000 random states fail, and removing the "alive with two neighbors" case made 926 of them fail. With the correct game.sv the count goes back to 0.